## Activity 19: Troubleshooting practical

Open Packet Tracer along with the ‘Packet Tracer Troubleshooting Activity.PKA’ file and apply your resolution.

A customer has reported the following issue and is looking for a speedy resolution: Users on Network1 unable to communicate with Network2.

Troubleshoot and resolve this issue.

Note: Remember to save any files once your activity is complete.

Topology

Consider the probable cause of failure (discuss possible scenarios that would create this problem):

Router misconfiguration,

Incorrect IP configuration on PC’s,

NO DHCP/DNS,

Faulty hardware.

List the actions that could possibly solve the problem:

ping PC11 from PC21 = no successful reply

Ping PC12 from Router IP = no successful reply

Ping PC14 from PC12= no successful reply

Did ipconfig on all PC’s in Network 1= got 169.254.212.12 which is APIPA. This is due to DHCP service failure.

So checked DHCP server for Service status and it was turned off. Hence needed to turn it on.

Also Identified Default Gateway misconfiguration to a different IP address i.e. 192.168.2.254 can change IP to 1.254.

Renewed IP on PC 11, and other PC’s in Network 1 checked with Ipconfig, and found valid configurations on all the PC’s.

ping PC11 from PC21 = no successful reply

Ping Router IP from PC11 = successful reply

Ping PC14 from PC12= successful reply

Check Router Configurations, and port settings. Identified Gigabyte Ethernet port 0/1 misconfigured to 3.254 instead of 2.254, so fixed that.

ping PC11 from PC21 = successful reply

ping PC12 from PC22 = successful reply

Verify the solution by repeating the tests from the above ticket.
(If the issue persists, undo any changes and return to your list):
ping PC11 from PC21 = successful reply

ping PC12 from PC22 = successful reply

Record your solution to the problem. (Your notes can prove invaluable if you encounter a similar issue again.):
Summarise how did you identify, what was the cause for the failure, how did you do fix it and how did you verify.